Description Phase: Specification, Planning, Design Entry, Functional Test.  Implementation Phase: Synthesis, Post-Synthesis Test, APR Parasitic Extraction & timing checks, Manufacture & Hardware Validation

HDL: 9x testing/validating

Enable Large Design, Portable Design, Explore Large solution space, Better Validated Designs, portable, self-documenting, describe multiple levels of abstraction, represent parallelism, provides many descriptive styles, Serve as input for synthesis tools.

can be compiled to semi-custom and programmable hardware implementations

Hardware Building Blocks

Transistors are switches

Use multiple transistors to make a gate

Use multiple gates to make a circuit

Standard Cells

Library of common gates and structures (cells)

Decompose hardware in terms of these cells

Arrange the cells on the chip

Connect them using metal wiring

FPGAs

“Programmable” hardware

Use small memories as truth tables of functions

Decompose circuit into these blocks

Connect using programmable routing

SRAM bits control functionality

Netlist

an ASCII text representation of the interconnect of a schematic

Port: can be bidirectional

Module Styles

Structural – connect primitives and modules

Dataflow– use continuous assignments

Behavioral – use initial and always blocks

A single module can use more than one method.

Structural

A schematic in text form (i.e. A netlist)

Build up a circuit from gates/flip-flops

Gates are primitives (part of the language)

Flip-flops themselves described behaviorally

reg [7:0] mem[0:99][0:3]; // 2D array of 8 bits

typedef enum reg [1:0] { IDLE, CONV, ACCM } state_t;

state_t state, nxt_state;
$display(“At time %t count = %h”,$time,cnt);

$stop  //Stops simulation and allows you to still probe signals and debug

$finish //completely stops simulation, simulator relinquishes control of thread.

Empty input ports => high impedance state (z)

Empty output ports => output not used

Inertial Delay (Gates)

Suppresses pulses shorter than delay amount 	In reality, gates need to have inputs held a certain time before output is accurate

Transport Delay (Nets)

“Time of flight” from source to sink

and #(1:2:3) g1(out,a,b);	// 1 ns min, 2ns typical, 3ns max delay

and #(2:3:4, 1:2:3) g1(out,a,b); // gate has different rise,fall for min:typ:max

$monitor

Output the given values whenever one changes; Can use when simulating Structural, RTL, and/or Behavioral

$display, $strobe

Output specific information like a printf in a C program; Used in Behavioral Verilog

Dataflow Verilog

The continuous assign statement

It is the main construct of Dataflow Verilog

It is deceptively powerful & useful

assign [drive_strength] [delay] list_of_net_assignments;

list_of_net_assignment ::= net_assignment [{,net_assignment}]

Net_assignment ::= net_lvalue = expression

===, !== test x’s, z’s!  ONLY USE FOR testbenches

Bitwise Operators (&, |, ^, ~)

Reduction XOR

assign parity = ^data_out; // even parity bit

When wire is used but not declared, it is implied

Latches with Continuous Assign

assign q_out = enable ? data_in : q_out;

Behavioral Verilog

initial and always form basis of all behavioral Verilog

All <LHS> assignments must be to type reg (or logic if using SV)

Behavioral: Combinational vs Sequential

Combinational

Not edge-triggered

All “inputs” (RHS nets/variables) are triggers

Does not depend on clock

Sequential

Edge-triggered by clock signal

Only clock (and possibly reset) appear in trigger list

Can include combinational logic that feeds a FF or registe

Blocking “Evaluated” sequentially  =

Used for combinational logic

The evaluation of subsequent statements <RHS> are blocked, until the <LHS> assignment of the current statement is completed.

Non-Blocking Assignments <=

“Updated” simultaneously if no delays given

Used for sequential logic

casez

Uses z and ? as “don’t care” bits in case items and expression

casex

Uses x, z, and ? as “don’t care” bits in case items and expression

Timing

Tsetup: Time D must be stable BEFORE clock edge

■ Adds to critical path delay

Clk->Q: Time from clock edge to Q changing

■ Adds to critical path delay

Thold: Time D must be stable AFTER clock edge

■ Sets minimum path from Q of one DFF to D of another

Cost(mapping) = -Slack(mapping) + Area(mapping) + [power(mapping)]

input delay is specified as time after the clock edge (of prior clock cycle) that the input to the DUT is valid.

Optimization Priorities

Design rules have priority over timing goals

Timing goals have priority over area goals

Max delay has priority over min-delay

To prioritize area constraints:

use the ignore_tns (total negative slack) option when you specify the area constraint:

set_max_area -ignore_tns 10000

Top-Down Compilation

Use top-down compile strategy for medium to small designs

Read in the entire design

Resolve multiple instances of any design references with uniquify

Apply attributes and constraints to the top level

Compile the design using compile or compile_ultra

Bottom-Up Compile Strategy

Compile the subdesigns separately and then incorporate them

Top-level constraints are applied and the design is checked for violations.

Advantages:

Compiles large designs more quickly (divide-and-conquer)

Requires less memory than top-down compile

Disadvantages

Need to develop local constraints as well as global constraints

May need to repeat process several times to meet design goals

Compile-once-don’t-touch Method

uses the set_dont_touch command to preserve the compiled subdesign

Ungroup Method

The ungroup command makes unique copies of the design and removes levels of the hierarchy

report_design

display characteristics of the current design

operating conditions, wire load model, output delays, etc.

parameters used by the design

report_area

displays area information for the current design

number of nets, ports, cells, references

area of combinational logic, non-combinational, interconnect, total

report_hierarchy

displays the reference hierarchy of the current design

tells modules/cells used and the libraries they come from

report_timing

reports timing information about the design

default shows one worst case delay path

report_resources

Lists the resources and datapath blocks used by the current design

repeat Loop

only evaluated when the loops starts

If it changes during loop execution it won’t change the number of iterations

forever loops

Only a $stop, $finish or a specific disable can end a forever loop

Sequential vs Parallel è (begin/end) vs (fork/join)

begin/end are used to form compound sequential statements.

fork/join are used to form compound parallel statements.

Statements in a parallel block are executed simultaneously.

All delay or event based control is relative to when the block was entered

Named Blocks

Blocks (begin/end) or (fork/join) can be named

Local variables can be declared for the named block

Variables in a named block can be accessed using hierarchical naming reference

Named blocks can be disabled (i.e. execution stopped)

disable Statement

Disables execution of the current block (not permanently)

由于Verilog不支持循环的continue和break，因此通过将最外层block命名为break，将循环里面的block命名为continue，通过disable continue和 disable break达成和C中continue与break同样的效果。

Assertions (only in System Verilog)

assert (true_condition) pass_statement

else fail_statement

assert (result == expected)  $display(“self check passed”)

else $fatal(“ERR: at time %t, result not same as expected”,$time);

$fatal è Throws a fatal message to output, exits the simulator (like a $finish).

$error è Throws a error message to output, continues simulation.

Assertions…immediate vs concurrent

The assertion condition is evaluated as the statement is encountered in the test bench flow.

Concurrent assertion

allows you to define conditions that should always be true, and are checked at all times during simulation

/// check that rd & wrt are never both asserted ///

/// This will be checked at every simulation tick ///

assert property (!(rd && wrt))

else $error(“ERROR: both rd and wrt asserted to SRAM”);

key word property distinguishes a concurrent assertion from an immediate assertion.

Most concurrent assertions would be checked on clock ticks.

assert property (@(posedge clk) req |-> ##[1:2] ack);

A ## followed by a number or range specifies the delay from the current clock tick to the beginning of the sequence that follows.

## is often used with a range.  For example: req|-> ##[0:3] gnt would mean gnt should be asserted 0 to 3 clock cycles after req.

$fopen opens a file and returns an integer descriptor

integer fd = $fopen(“filename”);

integer fd = $fopen(“filename”, “r”);

If file cannot be open, returns a 0

Can output to more than one file simultaneously by writing to the OR ( | ) of the relevant file descriptors

$fclose closes the file

$fclose(fd);

Writing To Files

Output statements have file equivalents

$fmonitor(), $fdisplay(), $fstrobe()

$fwrite()        // write is like a display without the \n

These system calls take the file descriptor as the first argument

$fdisplay(fd, “out=%b in=%b”, out, in);

Reading From Files

Read a binary file: $fread(destination, fd);

Can specify start address & number of locations too

Very rich file manipulation (see IEEE Standard)

$fseek(), $fflush(), $ftell(), $rewind(), …

Using $fgetc to read characters

reg signed [15:0] wide_char;

wide_char = $fgetc(file_handle);

Error

integer file_handle,error;

reg [639:0] err_str;

error = $ferror(file_handle,err_str);

Using $fgets to read lines

$fgets() returns the number of bytes in the line.  When this is a zero you know you hit EOF.

integer file_handle,error,indx,num_bytes_in_line;

reg [256*8:1] mem[0:255],line_buffer;

reg [639:0] err_str;

num_bytes_in_line = $fgets(line_buffer,file_handle);

Using $fscanf to read files

integer file_handle,error,indx,num_matches;

reg [15:0] mem[0:255][0:1];

reg [639:0] err_str;

num_matches = $fscanf(file_handle,”%h %h”,mem[indx][0],mem[indx][1]);

Loading Memory Data From Files

$readmemb(“<file_name>”,<memory>);

$readmemb(“<file_name>”,<memory>,<start_addr>,<finish_addr>);

$readmemh(“<file_name>”,<memory>);

$readmemh(“<file_name>”,<memory>,<start_addr>,<finish_addr>);

$readmemh Hex data…$readmemb binary data

Functions

Declared and referenced within a module

Used to implement combinational behavior

Contain no timing controls or tasks

Inputs/outputs

Must have at least one input argument

Has only one output (no inouts)

Function name is implicitly declared return variable

function [7: 0] aligned_word;           // function declaration

Tasks can modify global signals too

Task <taskname>; input…; output …; endtask

Tasks provide the ability to

Execute common procedures from multiple places

Divide large procedures into smaller ones

Include Compiler Directives

`include “adder8.v”  // include the tasks for adder8

A generate-loop permits making one or more instantiations (pre-synthesis) using a for-loop.

Only allowable uses of x is as “don’t care”

Only allowable use of  z:

Constructs implying a 3-state output

Min_delay_slack  =  clk2q – THold – Skew_Between_clks

1.) Use clock tree synthesis (CTS) within the APR tool to balance clock network

异步信号使用分开always block获得更小的面积

Translation (Elaboration)

Unrolls loops, substitutes macros & parameters, computes constant functions, evaluates generate conditionals

Builds a structural representation of the design

set compile_seqmap_enable_output_inversion true

Allows the mapping  of sequential elements in the design to library cells whose outputs are inverted.

The register retiming tool moves registers through the synthesized combinational logic network to improve timing and/or area

APR: Automatic Places & Route 布线用的

max path  The worst case (maximum) amount of time it takes to propagate from an input of a combinational block of logic to the output.

Clock skew There is uncertainty (a +/- margin) in a clock’s arrival time.

input delay is specified as time after the clock edge (of prior clock cycle)  that the input to the DUT is valid.

output delay is specified as time prior to next rising edge that the output must be valid.

MaxDelaySlack = ClockPeriod – clk2q – TMAX – TSU – clkSkew

MinDelaySlack = clk2q + TMIN – THLD – clkSkew

read_file -format verilog {./source/ss_a2d_dp.v}

# Define clock and set don't mess with it #

create_clock -name "clk" -period 20 -waveform { 0 10 }  { clk  }

set_dont_touch_network [find port clk]

# setup pointer that contains all inputs except clock #

set prim_inputs [remove_from_collection [all_inputs] [find port clk]]

# Set input delay & drive on all inputs #

set_input_delay -clock clk 5 [copy_collection $prim_inputs]

set_driving_cell -lib_cell AO33D0BWP -pin Z -from_pin A1 -library \

tcbn40lpbwptc [copy_collection $prim_inputs]

# tell it por_n strongly driven so it won't buffer

set_drive 0.1 por_n

# Set output delay & load on all outputs #

set_output_delay -clock clk 5 [all_outputs]

set_load 0.10 [all_outputs]

# Wire load model allows it to estimate internal parasitics #

set_wire_load_model -name TSMC32K_Lowk_Conservative \

-library tcbn40lpbwptc

# Max transition time is important for Hot-E reasons #

set_max_transition 0.10 [current_design]

# Now actually synthesize for 1st time #

compile -map_effort low

check_design

## design ware component caused extra pins

report_area

# Take a look at max & min timings #

report_timing -path full -delay max -nworst 3

report_timing -path full -delay min -nworst 3

## smash the hierarchy (design ware component)

ungroup –all

compile -map_effort medium

check_design

report_area

#### write out final netlist ######

write –format verilog ss_a2d_dp –output ss_a2d_dp.vg

# Sets maximum area to 20,000 area units (square microns)

set_max_area 20000

#Sets maximum dynamic power to 10 mW (default unit for our library is mW)

set_max_dynamic_power 10

ignore_tns (total negative slack) option specify the area constraint:

set_max_area -ignore_tns 10000

change priorities use set_cost_priority

set_cost_priority -delay

constraints: max_transition, max_fanout, max_capacitance, cell_degradation, max_delay, min_delay, power, area

compile options include:

-map_effort low | medium | high  (default is medium)

-area_effort low | medium | high (default same as map_effort)

-incremental_mapping (may improve already-mapped)

-verify (compares initial and synthesized designs)

-ungroup_all (collapses all levels of design hierarchy)

The ungroup command makes unique copies of the design and removes levels of the hierarchy

check_design command to verify design consistency.

check_timing to locate potential timing problems

report_contraints –all_violators (check everything)

Uniquify Treats every instance of a common sub module individually, so it can be optimized to its own constraints/context.

Ungroup Smashes (flattens) levels of hierarchy so optimizations can be made across boundaries.

Minimum clk period = Clk2q + Tmax + Tsetup

Min delay slack = Clk2q + Tmin – Thold

Input delay constraint set_input_delay -clock clk 1.2 Sig_out

Delay to input = Clk2q + Tmax

Average clock insertion delay = (mindelay + maxdelay)/2